Experiment No. 08

  - Experiment Name

8255 PPI interfacing: Dot matrix

  - Objectives

- To get acquainted with the "MDA 8086" Trainer Board and its operation

- To understand the operation of Dot matrix in 8255 PPI interfacing

- To learn how to implement program in “MDA 8086” Trainer Board and interconnect it with “Emu 8086”

  - Theory

A dot Matrix consists of an array of LED's which are inter connected such that the positive terminal (anode) of each LED in the same column are connected together and the negative terminal (cathode) of each LED in the same row are connected together.

Here, the 8255 port address in accordance with the dot matrix are as follows,

| Port name | Port address |
| --- | --- |
| Port A | 18H |
| Port B | 1AH |
| Port C | 1CH |
| Control register | 1EH |

  - Apparatus

- MDA 8086 - Trainer Board

  - Code for column wise operation

CODE SEGMENT

ASSUME CS: CODE, DS: CODE, SS: CODE, ES: CODE

CNTRL EQU 1EH

PORTC EQU 1CH

PORTB EQU 1AH

PORTA EQU 18H

ORG 1000H

MOV AL, 100000000B

OUT CONTRL, AL

MOV AL, 100000000B

OUT PORTB, AL

MOV AL, 000000001B

L1;

OUT PORTC, AL

ROL AL, 1

CALL TIMER

JMP L1

INT

TIMER:

MOV CX, 0FFFFH

LOOP2:

NOP

NOP

NOP

NOP

LOOP LOOP2

RET

CODE ENDS

END

  - Output for column wise operation

Fig. 8.1: Writing program on notepad

Fig. 8.2: Output

  - Code for row wise operation

CODE SEGMENT

ASSUME CS: CODE, DS: CODE, SS: CODE, ES: CODE

CNTRL EQU 1EH

PORTC EQU 1CH

PORTB EQU 1AH

PORTA EQU 18H

ORG 1000H

MOV AL, 100000000B

OUT CONTRL, AL

MOV AL, 11111111B

OUT PORTC, AL

MOV CL, 8

MOV AL, 01111111B

L1;

OUT PORTB, AL

ROL AL, 1

CALL TIMER

JMP L1

INT

TIMER:

MOV CX, 0FFFFH

LOOP2:

NOP

NOP

NOP

NOP

LOOP LOOP2

RET

CODE ENDS

END

  - Output for row wise operation

Fig. 8.3: Output

  - Discussion & Conclusion

The procedure of working in serial monitor mode or PC mode with the 8086 MDA kit was introduced in this experiment. We used code to perform the dot matrix. A program was developed in notepad and saved as ‘.asm’ file, which was then translated to ‘.obj’ which was then translated into a ‘.abs‘ file. This was then executed by following the instructions, and the output was observed using both single-step execution and direct execution.

By using programming, the row-wise LEDs were turned on after the column-wise LEDs. Therefore, it may be concluded from all of the foregoing talks that the experiment was a success.